Rev.1.2 Apr 23rd, 2019

E2-RX-FW Linux

Detailed Specification

www.swhwc.com

Table of Contents

1.	Overview	3

2.	Communication DLL to USB	4

2.1	API Functions	4

2.2	COM_Open	5

2.3	COM_Close	7

2.4	COM_SetTimeOut	9

2.5	COM_SendBlock	10

2.6	COM_ReceiveBlock	12

2.7	COM_ReceiveBlockH	13

2.8	COM_ReceiveBlockM	15

2.9	COM_ReceivePolling	16

2.10	COM_Return_data_size	17

2.11	COM_GetLastError	18

2.12	COM_Is_Open	19

2.13	COM_Lock	20

2.14	COM_UnLock	21

2.15	COM_SetTransferSize	22

2.16	COM_VendorControlIn	23

2.17	COM_VendorControlOut	24

3.	FFW	25

4.	NDEB App	26

4.1	Initialize Sequence (E2)	26

4.2	Target Memory Access	27

4.2.1	Target Memory Write (FFWMCUCmd_WRITE)	27

4.2.2	Target Memory Read (FFWMCUCmd_DUMP)	27

5.	Revision history	29

# Overview

This development migrates the RX-FW (for E2 Emulator) from Windows to Linux (Ubuntu).

# Communication DLL to USB

## API Functions

| Function | Description |
| --- | --- |
| COM_Open () | Initializes the communication interface |
| COM_Close () | Closes the communication interface |
| COM_SetTimeOut () | Receive timeout setting |
| COM_SendBlock () | Transmit block data |
| COM_ReceiveBlock () | Acquire received data (monitor serial only) |
| COM_ReceiveBlockH () | Acquisition of received data _ For obtaining large amount of data |
| COM_ReceiveBlockM () | Acquire received data (Receive response) For Masopi |
| COM_ReceivePolling () | Confirm existence of received data (E1usb.sys only) |
| COM_Return_data_size () | Confirm the number of received data |
| COM_GetLastError () | Obtain error number |
| COM_Is_Open () | Acquire communication status |
| COM_Lock () | Lock communication interface (reserved function) |
| COM_UnLock () | Unlock communication interface (reserved function) |
| COM_SetTransferSize () | USB communication transfer size setting |
| COM_VendorControlIn () | USB vendor control in |
| COM_VendorControlOut () | USB vendor control out |

Table 1 API Functions

## COM_Open

The function opens the communication interface.

Format

int COM_Open(long type, char *cparam, long lparam);

Parameters

type
Communication interface to open

cparam
Individual identification information (COM number or serial number) of the communication interface to be opened.
In this development, it does not support to connect multiple E2 to PC simultaneously. So this parameter will be ignored.

lparam
Setting the communication interface to open
Unused for E2.

Return Values

TRUE
Normal completion

FALSE
Abnormal termination

Description

The function opens the the communication interface.

Flowchart

Special Notes

None

## COM_Close

The function closes the communication interface.

Format

int COM_Close(void);

Parameters

None

Return Values

TRUE
Normal completion

FALSE
Abnormal termination

Description

The function closes the communication interface.

Flowchart

Special Notes

None

## COM_SetTimeOut

Receive timeout setting.

Format

int COM_SetTimeOut(long timeout);

Parameters

timeout
Timeout time (in seconds)

Return Values

TRUE
Normal completion

FALSE
Abnormal termination

Description

Receive timeout setting.

Special Notes

In this development, fixed timeout is used.

Just return TRUE.

## COM_SendBlock

Transmit block data (send command)

Format

int COM_SendBlock(char *buf, long size);

Parameters

buf 
Pointer to the area where received data is stored

long 
Number of bytes of data to be transmitted

Return Values

TRUE
Normal completion

FALSE
Abnormal termination

Description

Transmit block data (send command)

Flowchart

Special Notes

None

## COM_ReceiveBlock

Acquire received data.

Format

int COM_ReceiveBlock(char *buf, long size);

Parameters

buf 
Pointer to the area where received data is stored

size 
Receive buffer size (in bytes)

Return Values

TRUE
Normal completion

FALSE
Abnormal termination

Description

Acquire received data.

Special Notes

None

## COM_ReceiveBlockH

Acquisition of received data _ For obtaining large amount of data

Format

int COM_ReceiveBlockH(char *buf, long *size);

Parameters

buf 
Pointer to the area where received data is stored

size 
Pointer to store the number of received data

Return Values

TRUE
Normal completion

FALSE
Abnormal termination

Description

Acquisition of received data _ For obtaining large amount of data

Flowchart

Special Notes

None

## COM_ReceiveBlockM

Acquire received data (Receive response) For Masopi

Format

int COM_ReceiveBlockM(char *buf, long size);

Parameters

buf 
Pointer to the area where received data is stored

size 
Receive buffer size (in bytes)

Return Values

TRUE
Normal completion

FALSE
Abnormal termination

Description

Acquire received data (Receive response) For Masopi

Special Notes

Unused for E2.

Just return TRUE.

## COM_ReceivePolling

Confirm existence of received data (E1usb.sys only).

Format

int COM_ReceivePolling(long *arrive);

Parameters

arrive 
Stores presence / absence of received data. 
COM_EXIST: Received data exists, COM_NONE: No receive data

Return Values

TRUE
Normal completion

FALSE
Abnormal termination

Description

Confirm existence of received data (E1usb.sys only).

Special Notes

Unused for E2.

Just return TRUE.

## COM_Return_data_size

Confirm the number of received data.

Format

int COM_Return_data_size(int *size);

Parameters

size 
Pointer to the area storing the number of data

Return Values

TRUE
Normal completion

FALSE
Abnormal termination

Description

Confirm the number of received data.

Special Notes

None

## COM_GetLastError

Obtain error number.

Format

int COM_GetLastError(void);

Parameters

None

Return Values

Error number of the last error encountered

Description

Obtain error number.

Special Notes

This version of software does not return any error number. So No one use COM_GetLastError().

## COM_Is_Open

Acquire communication status.

Format

int COM_Is_Open(int *mode);

Parameters

mode 
Opened mode (the value of the type parameter of the COM_Open () function is stored)

Return Values

TRUE: 
Opened.

FALSE: 
Not open

Description

Acquire communication status.

Special Notes

None

## COM_Lock

Lock communication interface (reserved function).

Format

int COM_Lock(void);

Parameters

None

Return Values

TRUE
Normal completion

FALSE
Abnormal termination

Description

Lock communication interface (reserved function).

Special Notes

Reserved function.

Just return TRUE.

## COM_UnLock

Unlock communication interface (reserved function).

Format

int COM_UnLock(void);

Parameters

None

Return Values

TRUE
Normal completion

FALSE
Abnormal termination

Description

Unlock communication interface (reserved function).

Special Notes

Reserved function.

Just return TRUE.

## COM_SetTransferSize

USB communication transfer size setting.

Format

int COM_SetTransferSize(long size);

Parameters

size 
Specify transfer size (in byte units)

Return Values

TRUE
Normal completion

FALSE
Abnormal termination

Description

USB communication transfer size setting.

Special Notes

In this development, fixed transfer size is used.

Just return TRUE.

## COM_VendorControlIn

USB vendor control in.

Format

int COM_VendorControlIn(char req, unsigned short val, unsigned short index, char *in, int len);

Parameters

req 
Request of control transfer

val 
Value of control transfer

index 
Index of control transfer

in 
Input area

len 
Input area size

Return Values

TRUE
Normal completion

FALSE
Abnormal termination

Description

USB vendor control in.

Special Notes

Not implement in this development.

Just return TRUE.

## COM_VendorControlOut

USB vendor control out.

Format

int COM_VendorControlOut(char req, unsigned short val, unsigned short index, char *out, int len);

Parameters

req 
Request of control transfer

val 
Value of control transfer

index 
Index of control transfer

out 
Output area

len 
Output area size

Return Values

TRUE
Normal completion

FALSE
Abnormal termination

Description

USB vendor control out.

Special Notes

Not implement in this development.

Just return TRUE.

# FFW

Please refer to FFW_file_list.xlsx for the list of FFW API and modification points.

# NDEB App

## Initialize Sequence (E2)

At FFWE2RX startup, it is necessary to issue commands following the sequence described below.

Unless the sequence below is executed, command behavior cannot be guaranteed.

FFW Interface Issuing Sequence at Debugger Startup (FINE)

| Interface issuing No. | Interface name | Content | Remark |
| --- | --- | --- | --- |
| 1 | FFWE20Cmd_INIT | Initializes FFW module |  |
| 2 | FFWE20Cmd_GetEINF | Obtains emulator information |  |
| 3 | FFWE20Cmd_DCNF_xxx | Configures DCSF in emulator | Issued as necessary |
| 4 (*1) | FFWE20Cmd_EXTPWR | Supplies power to the user board | Issued as necessary between this step and step No. 6 |
| 5 | FFWE20Cmd_TRANS_EML | Goes to BFW EML mode |  |
| 6 | FFWRXCmd_SetMCU | Sets per-MCU information |  |
| 7 | FFWRXCmd_SetDBG | Sets debug information |  |
| 8 | FFWRXCmd_WTRLOAD | Loads internal flash write program |  |
| 9 (*2) | FFWE20Cmd_SetSCIBR | Sets emulator-to-MCU serial communication baud rate |  |
| 10 (*2) | FFWE20Cmd_SetSCICTRL | Sets emulator’s serial communication control information |  |
| 11 | FFWRXCmd_SetPMOD | Sets processor mode and endian (to specify startup mode) |  |
| 12 | FFWRXCmd_HPON | Connects with RX |  |
| 13 | FFWRXCmd_IDCODE | Sets ID code | Required only when internal flash ROM is enabled |
| 14 | FFWRXCmd_SetPMOD | Sets processor mode and endian (to check actual settings) |  |
| 15 | FFWMCUCmd_CLR | Initializes emulator’s internal settings |  |

*1 No need to issue when power is supplied externally.

*2 Since RX 231 does not have JTAG, it is necessary to set a baud rate for serial communication and serial communication control information.

## Target Memory Access

### Target Memory Write (FFWMCUCmd_WRITE)

[NDEB Command]

- Example 1: Write a file of incremental data from 00 to the address 0 to ffff

>write 00000000_0000ffff_inc_00.mot

- Example 2: Write incremental data from 0 to F to addresses 0 to f in byte size

>write 0,f,b,0,1,2,3,4,5,6,7,8,9,a,b,c,d,e,f

### Target Memory Read (FFWMCUCmd_DUMP)

[NDEB Command]

- Example 1: Read data of address 0 to ff in byte size

>dump b, 0,ff

ADDRESS    +0 +1 +2 +3 +4 +5 +6 +7   +8 +9 +A +B +C +D +E +F   ASCII

00000000   00 01 02 03 04 05 06 07   08 09 0A 0B 0C 0D 0E 0F   ................

00000010   10 11 12 13 14 15 16 17   18 19 1A 1B 1C 1D 1E 1F   ................

00000020   20 21 22 23 24 25 26 27   28 29 2A 2B 2C 2D 2E 2F    !"#$%&'()*+,-./

00000030   30 31 32 33 34 35 36 37   38 39 3A 3B 3C 3D 3E 3F   0123456789:;<=>?

00000040   40 41 42 43 44 45 46 47   48 49 4A 4B 4C 4D 4E 4F   @ABCDEFGHIJKLMNO

00000050   50 51 52 53 54 55 56 57   58 59 5A 5B 5C 5D 5E 5F   PQRSTUVWXYZ[\]^_

00000060   60 61 62 63 64 65 66 67   68 69 6A 6B 6C 6D 6E 6F   `abcdefghijklmno

00000070   70 71 72 73 74 75 76 77   78 79 7A 7B 7C 7D 7E 7F   pqrstuvwxyz{|}~.

00000080   80 81 82 83 84 85 86 87   88 89 8A 8B 8C 8D 8E 8F   ................

00000090   90 91 92 93 94 95 96 97   98 99 9A 9B 9C 9D 9E 9F   ................

000000A0   A0 A1 A2 A3 A4 A5 A6 A7   A8 A9 AA AB AC AD AE AF   ................

000000B0   B0 B1 B2 B3 B4 B5 B6 B7   B8 B9 BA BB BC BD BE BF   ................

000000C0   C0 C1 C2 C3 C4 C5 C6 C7   C8 C9 CA CB CC CD CE CF   ................

000000D0   D0 D1 D2 D3 D4 D5 D6 D7   D8 D9 DA DB DC DD DE DF   ................

000000E0   E0 E1 E2 E3 E4 E5 E6 E7   E8 E9 EA EB EC ED EE EF   ................

000000F0   F0 F1 F2 F3 F4 F5 F6 F7   F8 F9 FA FB FC FD FE FF   ................

- Example 2: Read data of address 0 to 3f in word size

>dump w,0,3f

ADDRESS    +1+0 +3+2 +5+4 +7+6   +9+8 +B+A +D+C +F+E   ASCII

00000000   0100 0302 0504 0706   0908 0B0A 0D0C 0F0E   ................

00000010   1110 1312 1514 1716   1918 1B1A 1D1C 1F1E   ................

00000020   2120 2322 2524 2726   2928 2B2A 2D2C 2F2E   ! #"%$'&)(+*-,/.

00000030   3130 3332 3534 3736   3938 3B3A 3D3C 3F3E   1032547698;:=<?>

- Example 3: (Read data of address 0 to 3f in lword size)

>dump l,0,3f

ADDRESS    +3+2+1+0 +7+6+5+4   +B+A+9+8 +F+E+D+C   ASCII

00000000   03020100 07060504   0B0A0908 0F0E0D0C   ................

00000010   13121110 17161514   1B1A1918 1F1E1D1C   ................

00000020   23222120 27262524   2B2A2928 2F2E2D2C   #"! '&%$+*)(/.-,

00000030   33323130 37363534   3B3A3938 3F3E3D3C   32107654;:98?>=<

# Revision history

| Rev. | Date | Description |  |
| --- | --- | --- | --- |
|  |  | Page | Summary |
| 1.0 | Feb 22nd, 2019 |  | Add Communication DLL to USB |
| 1.1 | Feb 27th, 2019 | 4, 12, 18 | Fix some mistakes Add support for COM_ReceiveBlock Add special note for COM_GetLastError |
| 1.2 | Apr 23rd, 2019 | 26-28 | Add 4. NDEB App |